[image: image1.png]GHilii



[image: image2.png]


[image: image3.png]13 [0.512°]

Couping

26 [1. nzmx

14 [0.551]

l

Input

110 [4.331]

Output

PTveE
N-Female
(3IPLACES

20 It






Directional Coupler 

GHDC10N-0.7-2.5G
Applications

. Wireless Communications            . Military Communications

. SATCOM                            . Commercial Communications

. Test and Measurement               . including as electronic warfare (EW)
Description

The GHDC10S-0.7-2.5G is a 10dB directional coupler that operates from 0.7 to 2.5GHz and can handle up to 100 Watts(CW)with 20 dB directivity minimum and 1.3:1 VSWR maximum.
Couplers are specifically developed for system applications that require external leveling, accurate monitoring, signal mixing, or swept transmission and reflection measurements. 

For various applications, including as electronic warfare (EW), commercial wireless, SATCOM, radar, signal monitoring and measuring, antenna beam shaping, and EMC testing conditions, these couplers offer straightforward solutions.
Function

.Low insertion loss and VSWR.

.They are available in both narrow and broad bandwidths and can be used in a wide variety of applications including signal sampling,VSWR measurements,power combining and distribution. 
Electrical Specifications

	Model
	GHDC10N-0.7-2.5G

	Frequency Range (GHz)
	0.7 ～ 2.5

	Coupling (dB)
	10±0.5

	Frequency Sensitivity(dB)
	N/A

	Directivity(dB)
	≥20

	Insertion Loss (dB)
	≤1.0

	VSWR
	≤1.30 : 1

	Power Handling(Watt)
	≤100（CW）

	Operating Temperature(℃)
	-25 ～ +65

	Impedance
	50Ω

	Port Connectors
	N–Female

	Surface Color
	White

	Outline  Drawing
	As Below



Goghel Technology Co., Ltd.               Http:www.GOGHEL.com   Sales@goghel.com 


